Master-Slaves RPI system for AV acquisition

- General info

.pro = .pro files are project configuration files typically associated with Qt, a popular C++ framework used for developing cross-platform applications with graphical user interfaces (GUIs). The .pro file is part of the qmake build system, which is a tool used to automate the generation of Makefiles or other build scripts for building the project

.cpp = C++ files, typically the source files used inside the project, leading the application developpement and functioning

- Main.cpp

- Secondary .cpp files

.h = header files. They typically contain declarations for functions, variables, and macros, as well as data type definitions and constants. They serve as interfaces between different parts of a program, allowing functions or classes defined in one file to be accessed by other files without the need to redefine them

.o = object files, are compiled, machine-readable files generated from source code files (such as .c or .cpp files in C or C++). These files contain the binary code produced by the compiler after translating human-readable code into machine instructions, but they are not yet fully executable. Object files are an intermediate step in the process of building a final executable program.

.ts = In the context of the Qt framework, a .ts file is an XML-based file that contains translations for internationalization (i18n) and localization (l10n) of Qt applications. These .ts files are used by the Qt Linguist tool to manage and edit translations of text strings used in the application

.ui = XML-based files used in the Qt framework to define the layout and structure of graphical user interfaces (GUIs). These files are typically created using Qt Designer, a tool that allows developers to design GUIs visually by dragging and dropping widgets, without writing code directly. The .ui file stores the description of the interface in XML format, specifying which widgets (buttons, labels, input fields, etc.) are present and how they are arranged

.bmp = short for Bitmap Image File or Bitmap (often referred to as Device Independent Bitmap (DIB) file format), is a raster graphics image file format used to store digital images. It is one of the simplest image formats and is widely supported across different platforms and applications.

Psychophysical master

PsychophysicalMaster should run on a RPI (the master) connected to a network that is accessible by the RPIs running RunMatrix (the slaves) and the laptop running PsychophysicalInterface. The master will receive the experimental json from PsychophysicalInterface and execute the experiment.

config.json sets various parameters. The RPI this is run on needs to be able to accept connections over the network from all clients.

A breadboard needs to connect:

- Common grounds for all RPIs

- The audio outputs (AUDIO_PORTS) to an amplifier

- The TRIAL_RESET_PORT and TRIAL_START_PORT on the master to all slaves running RunMatrix, controlling the LEDmatrix

On each trial, it will:

- Load experiment information from the network (PsychophysicalInterface) if USE_INTERFACE_NOT_FILE is set in config.json

- Send a signal (TRIAL_RESET_PORT) that the trial is resetting

- Inform (via network) the slave RPIs running RunMatrix as to visual information

- Confirm that the slave RPIs have connected (communication by terminal)

- Send a signal (TRIAL_START_PORT) triggering the trial start to the slave –-> slave will start the trial and send data to the matrices

- During the trial, it will trigger GPIOs to play audio signals (claims the ports in AUDIO_PORTS, plays based on the experiment JSON). These ports need to be routed through a simple amp.

- It will also record key presses as a simple response measure. --> how?

- inform and distribute pixel information to the RunMatrix RPIs, control the auditory output. It is also set to record keypresses as a response measure and record timestamps.

Uses the lgpio library to control the GPIOs.

A few other notes:

Make sure the RPIs have sufficient amps from the power supplies.

Auditory functionality is very limited -- think about what you really want prior to the experiment.

The networking code here is very simple, an asynchronous method may be preferable.

I was using a confirmatory signal from the slaves to the master via SLAVE_IMAGE_READY_PORTS, but in testing this doesn't seem to be required, so I dropped it.

Psychophysical interface

This program is run on a laptop that connects to a controller running PsychophysicalMaster.

The application provides a GUI for configuring and uploading experiments to PsychophysicalMaster (i.e. to the server/master RPI). Each experiment that is configured is transmitted in JSON format. The GUI allows you to set experimental parameters (e.g., subject name), add Visual and Auditory trials, edit or delete created trials, or "mix" (combine) created trials into a new trial that contains the stimuli of the mixed/combined trials (useful for creating visual-auditory, visual-visual, and auditory-auditory trials). Note that mixing creates a new trial, it does not create pointers to the existing trial, so modifications on the mixed trial are not backpropagated.

The GUI has a field for the IP address of the CPU (i.e., a RPI) running PsychoPhysicalMaster, no other configuration is necessary. It connects via Port 13 using simple code (that could be improved).

PsychophysicalInterface.pro: configuration file of the folder, creates link between compilables files.

- Configures std C++ version

- Includes paths

- Calls sources (.cpp files)

- Includes libraries (.h files)

- Calls .ui files for the GUI

- Allows translation to US english

- Defines deployment tasks based on operating system

ExperimentStub.cpp: returns .json file with info on the experiment:

- Subject name

- Number of loops

- Trial ordering

- Write output (T/F)

- Serialises trials

Runmatrix

RunMatrix runs on a RPI controlling LED Matrices (several may be chained)

Requires installation of rpi-rgb-led-matrix and the "Adafruit hat" to be configured for "optimal" performance (requires soldering)

Run this program under sudo with argument: --led-slowdown-gpio==3

Connects to PsychophysicalMaster over an ethernet connection. Simple and effective solution is to connect via router/access point (to minimize interference).

config.json contains necessary configuration details.

Order of operations goes:

- RunMatrix enters STANDBY state listening to TRIAL_RESET_PORT

- PsychoPhysicalMaster will switch TRIAL_RESET_PORT to 1 and open a socket

- RunMatrix connects to socket and downloads experiment json

- RunMatrix listens to TRIAL_START_PORT

When triggered, RunMatrix plays the visual information on that trial

Main.cpp

The main project file, in the sense of the code that executes the program. The file containing the main() function, the entry point of the program, where execution begins

- RPI set up and Master-Slave connection: description

Follow instructions to set up RPIs

https://www.raspberrypi.com/documentation/computers/getting-started.html

In this file I will address

Connection to implement

The RPIs communicate with one another in two ways:

- Physical connection: specific GPIOs plugged into one another, which you can do with “fly leads”. The GPIO connections control the running of trials

- Network: they connect over a network. You can either set the master as a wireless access point or (perhaps preferably) plug them all into a physical access point/router via ethernet. The network connection allows the slave RPIs to download image files from the master.

the basic logic is this:

the master RPI controls multiple things:

- Receiving information from a laptop running the Interface program (PsychophysicalInterface)

- Updating the visual information for the slave RPIs,

- Controlling the auditory output,

- Running the basic experimental loop.

To describe this process as a Finite State Machine (FSM), we need to break it down into states, transitions, and events. Each state represents a phase of the system, and transitions occur based on specific events or conditions

States:

- Idle: The system is waiting for the trial to start.

- Waiting for Reset: The master and slaves are waiting for the TRIAL_RESET_PORT to be high.

- File Distribution: The slaves are connecting to the master, downloading files, and getting ready for the trial.

- Waiting for Trial Start: The system is ready and waiting for the trial to begin (i.e., the TRIAL_START_PORT to be high).

- Trial Running: The master and slaves are playing the trial files, and the master is sending PWM signals and recording keypresses.

- Finish Trial: The trial ends, and the system transitions back to the reset state to start the next cycle.

Key Points:

- Synchronization: GPIO ports (like TRIAL_RESET_PORT and TRIAL_START_PORT) synchronize the master and slaves.

- Network Communication: The master distributes files over Ethernet to the slaves.

- PWM Outputs: The master plays auditory stimuli during the trials by controlling AUDIO_PORTS via PWM signals.

- Looping: After each trial, the system resets and prepares for the next trial.

Transitions and Events:

- Idle → Waiting for Reset:

- Event: System initialization or completion of a previous trial.

- The system enters the reset state, waiting for the master to raise the TRIAL_RESET_PORT.

- Waiting for Reset → File Distribution:

- Event: TRIAL_RESET_PORT goes high.

- Slaves connect to the master over the network (ethernet connection) to download new files.

The master switches the TRIAL_RESET_PORT to high and opens sockets for the slaves. The slaves listen for that TRIAL_RESET_PORT. When it is high, they connect to the master over the ethernet connection and download new files.

- File Distribution → Waiting for Trial Start:

- Event: Files are successfully downloaded to all slaves.

- The system is now ready for the trial to begin, waiting for the TRIAL_START_PORT.

- Waiting for Trial Start → Trial Running:

- Event: TRIAL_START_PORT goes high.

- The master triggers the slaves to start playing their files. The master also begins sending PWM signals to play auditory stimuli and records any keypresses.

This is the state in which it plays trials. This triggers the slaves to play their files. During the trial-running state, the master also sends in parallel PWM outputs to the Elegoo through AUDIO_PORTS as appropriate to play auditory stimuli. It also records keypresses.

- Trial Running → Finish Trial:

- Event: Trial ends --> when? What defines the end of each trial?

- The system switches to the finish state, ready to reset for the next trial.

- Finish Trial → Waiting for Reset:

- Event: Master switches TRIAL_RESET_PORT high again to reset the state for the next trial

- The cycle restarts.

Phases of the Socket Connection:

- The master sets TRIAL_RESET_PORT to high:

- When the master sets the TRIAL_RESET_PORT to high, it signals to the slaves that they need to prepare to connect and download new instructions or data.

- The slaves are monitoring this port, waiting to receive the signal.

- Slaves detect the TRIAL_RESET_PORT signal:

When a slave detects that the TRIAL_RESET_PORT is high, it triggers the logic that establishes a connection to the master through a TCP/IP socket.

- Opening TCP/IP sockets:

- The master, after setting TRIAL_RESET_PORT to high, opens one or more TCP sockets to accept connections from the slaves. A socket is an endpoint for communication, defined by an IP address and a port number.

- At the same time, the slaves, which already know the master's IP address and the port to connect to (either defined in the configuration file or hardcoded), start requesting a TCP connection.

- Socket connection between the master and slaves:

- When a slave attempts to connect to the master, the master accepts the connection, establishing a bidirectional socket session. The master can have multiple sockets open to handle multiple slaves simultaneously.

- Through the socket connection, the master sends new files or images that the slaves will need to use during the upcoming trial.

Typical Flow of a TCP Socket Connection:

Master (Server):

- The master is set up as a server, opening a listening socket (bind) on a predefined port number.

- For example, the master might be listening on port 1234 for TCP connections from the slaves.

Slave (Client):

- Each slave acts as a client, attempting to connect to the master’s IP address and the open port.

- When the client (slave) detects that TRIAL_RESET_PORT is high, it initiates the connection request (connect) to the master.

Data Exchange:

- Once the connection is established, the master sends the necessary data, such as images or configuration files. The slaves receive and process this data, storing it locally for future use.

- This process occurs through a TCP data stream, ensuring that files are transferred reliably and in order.

Closing the Connection:

At the end of the transfer, the socket can be closed by either the master or the slave. Alternatively, the socket can remain open for future communications during the experiment.

Configuration json file for reassignment of the ports that are used for different purposes, RPI master side.

{

"DEFAULT_TRIAL_DURATION" : 3000,

"VISUAL_CONTROLLER_NAMES" : ["SLAVE1"],

"USE_INTERFACE_NOT_FILE" : false,

"EXPERIMENT_FILE_IF_USED" : "sample_experiment.json",

"TRIAL_START_PORT" : 10,

"TRIAL_RESET_PORT" : 19,

"SLAVE_IMAGE_READY_PORTS" : [5],

"AUDIO_PORTS" : [6]

}

Configuration json file for reassignment of the ports that are used for different purposes, RPI slaves side.

{

"SERVER_ADDRESS": "10.42.0.1",

"SERVER_PORT": "13",

"MY_NAME" : "SLAVE1",

"OUTPUT_IMAGE_READY_PORT": 9,

"INPUT_TRIAL_ON_PORT": 10,

"INPUT_TRIAL_RESET_PORT": 19,

"MATRICES_CHAINED":2

}

- GPIO 10 (MOSI port) from master (TRIAL_RESET_PORT - output) to breadboard --> to GPIO 10  (MOSI port) of each slave (INPUT_TRIAL_ON_PORT - input) [BROWN]

- GPIO 19 from master (TRIAL_START_OPORT - output) to breadboard --> to GPIO 19 of each slave (INPUT_TRIAL_RESET_PORT - input) [ORANGE]

- GPIO 6 from master (AUDIO_PORT - output) to breadboard --> poi? [VIOLET]

Other options for connection

Aim:

Our goal here is to find a way to implement an efficient connection in a master/slave mode for 3 different components. We need to build a system in which one master Pi board controls two different and parallel slave Pi. And these slave boards are sending real-time data back to master. The Pi master and Pi slaves must also be connected via a dedicated (wireless) network.

Components:

- Raspberry PI – 4 model B (x 3)

- Adafruit RGB matrix bonnet for Raspberry Pi (x 2)

- Jump wires

- Breadboard

Physical communication:

The design of the communication architecture involves three devices (the three Pi boards) in a 2 levels communication system: 1 leader component and 2 symmetric follower components:

For an efficient direct hardware connection, we have considered either SPI or I2C protocols. Each has its own advantages and limitations.

1. SPI Protocol

SPI (Serial Peripheral Interface) is a widely used serial communication protocol for devices connections and data transmission. It is ideal in case one must work with a single leader and several follower devices.

SPI is a full-duplex protocol, meaning that we can transfer data between devices in both directions (leader to followers and backwards) simultaneously on the channels.

- Requires a dedicated Chip Enable (CE/CS) pin for each follower device. If you have many leaders, this can become complex.

- Generally faster, can reach speeds of several megahertz (up to 10 MHz or more), making it ideal for applications that require high data transfer rates.

- Requires at least three wires (MOSI, MISO, SCLK) and one for each follower (CE/CS), increasing the number of connections.

- Useful for more occasional communications, such as controlling sensors and peripheral devices.

- Does not natively support multi-leader communication, and error management is simpler.

Works better over short distances (up to a few decimeters), as signal quality can degrade over longer distances.

2. I2C Protocol

I2C (Inter-Integrated Circuit) is the protocol for several devices’ communication based on a two-wire bus architecture. It is and actual half-duplex protocol, where the primary device controls the communication, and the secondary devices follow the master's instructions, meaning that data transmission can happen in just one direction at a time (leader cannot simultaneously receive and transmit)

- Can manage multiple devices (up to 127) on a single bus using unique addresses. It is better suited if you have many devices to connect.

- The standard speed is 100 kHz, but it can reach up to 400 kHz (or 1 MHz in Fast Mode), but it is still slower than SPI.

- Uses only two wires (SDA and SCL) plus ground, simplifying wiring, especially in complex circuits.

- Useful for more occasional communications, such as controlling sensors and peripheral devices

- Supports communication with multiple masters and can manage errors through ACK/NACK, making it robust for more complex applications.

- More suitable for longer connections (up to a few meters), thanks to pull-up resistors that keep the signal stable.

Okay now say we’d like to implement the I2C? How is this done practically?

SDA = SDA refers to the Serial Data Line in the I²C (Inter-Integrated Circuit) communication protocol.

SCL = SCL stands for Serial Clock Line in the I²C (Inter-Integrated Circuit) communication protocol

SCL provides the clock signal needed to synchronize data transfers between the master (Raspberry Pi) and the slave devices. The clock signal is generated by the master to control the timing of data exchanges on the SDA line.

- GPIO 2 master (which corresponds to SDA master) connected to bread (green) -> to SDA of each fRPIs

- GPIO 3 master connected to bread (yellow) -> to SCL of each fRPIs

All RPIs should be connected to the same ground on the GND breadboard line (blue one)

- GND of LRPI = pin 9 (blue)

- GND of fRPI = gnd on each bonnet (blue)

- LAN implementation and configuration – theory

This first paragraph addresses the basing principles beyond the implementation of a Local Area Network (in parallel to the analog signaling through GPIO ports) as a suitable, stable, quick and reliable system of communication (transmission of data) among different devices that must be synchronized within a unique experimental pipeline. By theory these network can be implemented with (ethernet) or without (wirelessly) cables.

- Working principles

- Physical connections implementation

- Cables

- Wireless

- Configuration

- Workflow of communication

WORKING PRINCIPLES

The network has the role to implement a fast and efficient connection between:

- the laptop (device leading the user interface program, through which we control all the trial parameters)

- the master RPI (the one running PsychophysicalMaster), receiving the commands for the current trial from the laptop and controlling al the stimulation process through the following devices (slave RPI, Elegoo, amplifier)

- the 2 slave RPI (each running RunMatrix) responsible for the control of the led matrix visual stimuli control

Network role:  allow transmission of DATA among the listed devices, while all the other aspects of the communication (signals of control over the state of the trial procedure - synchronization, readiness, start, reset, waiting etc) are entrusted to the GPIOs connection side.

PHYSICAL CONNECTION IMPLEMENTATION

To implement a network connection between your laptop, the Raspberry Pi master (SERVER), and the two Raspberry Pi slaves (CLIENTS), you can use either a wired (Ethernet) or a wireless (Wi-Fi) connection.

Both methods allow you to set up a local area network (LAN) between your laptop, Raspberry Pi master, and Raspberry Pi slaves for tasks like file transfers, SSH access, and experiment control

SSH protocol (not necessary at ur aims)

SSH (Secure Shell) is a network protocol that allows remote access via an encrypted connection. It provides robust authentication and supports secure remote access, command execution, file transfer, access control, TCP/IP forwarding, etc.

You can manage your files and folders via an SSH connection, change their permissions, edit files directly on the server, etc. This secure protocol creates a secure channel between a local computer (laptop) and a remote server (master RPI), allowing commands to be executed, resources to be accessed, files to be transferred, performance updates, etc. It is also used in various file transfer protocols, routers, server hardware management, and operating system administration.

Option 1: Wired Network (ETHERNET)

This setup connects all devices via Ethernet cables to a router or network switch. The router can be connected to the internet, but it's not necessary for local communication, it’s only necessary to have 4 Ethernet free ports.

Yu can see al details and configuration steps both via hardware and software in the doc: Setup and connections USA

Option 2: Wireless Network (Wi-Fi)

1. Raspberry Pi Master as a Wireless Access Point

You can configure the Raspberry Pi master to act as a Wi-Fi access point (WAP), allowing the Raspberry Pi slaves and laptop to connect wirelessly (no Ethernet cables needed). We refer to this type of network as WLAN (wireless local area network).

Steps:

- Set up the Raspberry Pi master as a wireless access point:

The master RPI will work as a router without internet connection, which is useless for this purpose: we only have to transfer data wirelessly between laptop/master RPI/slaves RPIs.

No cables needed in this configuration: the only necessary thing is the personal IP address of each device so that they can find the others in the network, and establish a connection

- Configure the Raspberry Pi slaves and laptop to connect to the new Wi-Fi network broadcasted by the Raspberry Pi master.

- Assign static IP addresses or use networkManager (a system network service that manages your network devices and connections, attempting to keep active network connectivity when available. It manages ethernet, WiFi etc.)  to automatically assign IP addresses.

- Test the connection:

Use ping or ssh from the laptop to the Raspberry Pi slaves and master to ensure all devices are connected to the network.

2. Connecting to an Existing Wi-Fi Network

If you already have a Wi-Fi network (such as your home router), you can connect all devices to the same Wi-Fi network.

Steps:

- Connect the Raspberry Pi master, Raspberry Pi slaves, and laptop to the existing Wi-Fi network.

- Check IP addresses:

If the Wi-Fi router has a DHCP server, it will assign IP addresses to each device.

You can check the assigned IP addresses using ifconfig on the Raspberry Pis or in the network settings on your laptop.

- Communicate between devices:

You can use SSH or other communication protocols to interact between devices.

For example, to connect to the master Pi from the laptop, use ssh pi@<IP of master>.

LAN code workflow of the actual communication

In this second paragraph we describe the actual implementation and workflow of the network as we set in the actual project of our psychophysical experiment setup. The hardware implementation of the network (an ethernet cabled one) between master and one slave is described above. I will add a description of how to connect the laptop to the same network. Moreover, the single steps of the process for each side (laptop and master firstly, master and slave secondly) will be discussed following the specific functions of the code that have been written. Also, gradually emerging issues and resolved problems will be highlighted.

Una volta impostata la rete funzionante, come viene sfruttata durante un generico trial, da inizio a fine, quali dati passano da laptop a rpiM, e da rpiM ai rpiS

Spiega le informazioni di gestione della comunicazione dove si trovano nei vari codici e come i codici gestiscono il flusso dei dati in I/O

- CONFIGURING CONNECTION

- Configure each RPI based on the role (master or slave) and libraries required for code to run correctly in each of the two

- Connecting the device to a monitor, mouse, and keyboard

- Configuring network settings to manage a link via the Ethernet port to a physical local area network

- Implement the hardware connection between master and slave, therefore through the physical ports of GPIO, and of the manual Local Area Network (LAN) changed, i.e. via Ethernet cables, which will be the ones used to connect the laptop from which to control the GUI through a network switch. Each device will have its own static IP address.

Materials needed:

- RPI master

- RPI slave1 (with bonnet and 2 matrices connected)

- USB-C power adapter for each RPI

- Bonnet power supply

- 2 ethernet cables (RJ45 connector)

- A network switch with at least 3 LAN ports (it doesn't matter if it's a router, we only make physical connections anyway) + its power supply

- One HDMI-to-micro HDMI cable (or with an adapter) for each RPI (to connect to the monitor)

- One mouse, keyboard, and monitor with HDMI input for each RPI

Note: If your monitor only has a VGA input, you will also need a VGA-to-HDMI adapter

- Jumpwires e breadborard

Tip: find a large and comfortable space in which to make the various connections because there are a lot of cables to keep at bay, plus two keyboards and the two monitors!

1. Connections between devices

Place the bonnet on the slave pins.

The ports we will have to use are in the config.json files of both projects.

On the slave side we will have to use:

"OUTPUT_IMAGE_READY_PORT": 9, à door with the word MI on the bonnet

"INPUT_TRIAL_ON_PORT": 10, à door with MO written on the bonnet, attached to the E

"INPUT_TRIAL_RESET_PORT": 19, à is at the bottom

On the master side we will have to use:

"TRIAL_START_PORT" : 10, à will be connected to port 10 of the slave

"TRIAL_RESET_PORT" : 19, à will be connected to port 19 of the slave

"SLAVE_IMAGE_READY_PORTS": [5], à will be connected to port 9 of the slave

"AUDIO_PORTS": [6] à will be connected to the breadboard to control the audio output (for now we don't consider it)

- Use the same colors for the corresponding ports.

- Connect the breadboard ports one at a time: each row of the breadboard is all at the same potential, so e.g., row 22 insert the wires of port 10 of the master and port 10 of the slave, row 20 port 19 master and port 19 slave etc. for all 3)

- Important! The two must also have the same ground, so on the breadboard connect  to the blue line (-) two GND terminals of both RPIs (there are more than one on each as you can see from the image of the pins)

- Also connect the ethernet cables to each RPI in the meantime, and connect them to two ports on the network switch.

- NB: the cables that we had put at the SDA, SCL ports are not needed. You must have 4 wires in total for each rpi: 1 GND, and 3 ports for signals.

2. Power on and monitor

- Connect both RPIs to their respective monitor (to micro HDMI input), mouse and keyboard (to two USB inputs)

- Connect power supplies to rpi (USB-C input)

- Connect power supply to bonnet

- Connect power supply to network switch

- Turn on everything: monitor, switcher, and the two RPI

You will now have direct control over each of the two RPIs: if you open the terminal in each you should read what they are called, with the type "pi@[name]". Ben called them

- Slave: cuppinislave1

- Master: "cuppinimaster"

3. Network Configuration

We will not use any wireless network, we will do everything by cable. More stable, faster, safer.

If you went to the master you could see at the top where there is the connection symbol if you click on it, that its hotspot like "cuppinimasterwifi" is active. We don't need it and you can turn it off:

- From the drop-down menu of connections go to "Advanced options" then to "Edit connections..."

- Select the hotspot network

- Click on the minus below

- Click on "Delete"

Create the LAN:

Now the RPIs are connected but still not communicating with each other so we have to create a network and assign a (static) IP address to each one. Static because every time we turn them back on they are automatically found.

This procedure must be done identically on both the master and the slave, ONLY the IP address we create for each one changes

- Go to the drop-down menu of the network connection à "Advance Options" à "Edit connections..."

- Click on the +

- Select "Ethernet"

- Under "Device" write "eth0", it will be the name of the interface, for both devices

- Go to the IPv4 section à select Method: Manual

- Click on "Add" and enter these IPs

For the slave I put: 192.168.1.11

For the master's degree I put: 192.168.1.10

Netmask value: 24 for both

Leave the Gateaway empty

- Click Save and close

Open Terminal

- Run: sudo nano /etc/hostname

- If it is not already written, enter: "cuppinimaster" (for the master) and "cuppinislave1" for the slave

- To save and exit: Ctrl+X to Y to Enter

- Run: sudo nano /etc/hosts

- At the bottom of the lines, add one and write

< IP of the slave> cuppinislave1 à for the slave

< IP of the master> cuppinimaster à for the master

- Per salvare e uscire: Ctrl+X to Y to Enter

Restart your device. On the terminal type: sudo reboot

When you have restarted, do a check. Go to the network settings at the top right, from the drop-down menu go to Advance options à Connection information...

You should see the "Ethernet connection 1" network which is our LAN, both on the master and slave.

A further test is to go to the terminal and write: ping < IP of the other device> à if everything goes well you should see a series of packets that are sent without problems.

Press Ctrl+C to finish

4. Organization of files and libraries

Su slave à directory /home/pi:

- A "projects" folder with the entire RunMatrix folder inside

- The rapidjson-master folder (library)

- The asio-1.30.2 folder (library)

- The rpi-rgb-led-matrix folder (library)

Su master to directory /home/pi:

- The PsychophysicalMaster folder

- La cartella di lg (library: https://github.com/joan2937/lg)

- The asio-1.30.2 folder (library)

- The rapidjson-master folder (library)

5. Execution

Now open Qt on both monitors.

- On RPI slave open the project RunMatrix.pro

- On RPI Master open the project PsychophysicalMaster.pro

- go to the RunMatrix "config.json" file à change the SERVER_ADDRESS with the one you set for the master in the LAN (as I wrote it is "192.168.1.10")

- go to the "experiment.cpp" file of the master, at lines 130-133 there should be a commented part. Make sure it's commented on:

// THIS IS FOR BYPASS

switchTstate(T_TRIAL_ON);

// continue;

- Do the built for both (hammer sign)

- ERRORE: (is object...) detick shadow build

- run first  the RunMatrix then the PsychophysicalMaster. - this doesn't work, but it works the other way around, i.e. running the master first and then the runmatrix

It should be over here, I don't remember if there were other essential changes to be made to the codes, you try to start them then let's see what errors come out.

- Setup and connections: master-amplifier-2 speaker audio

Implement the audio control connection: master > audio amplifier > speakers

Materials and methods:

- Master RPI

- 2 speaker wire to rca adapter

- Speaker cables

- 2 speakers

- Audio amplifier 2 channels

The connection for controlling the audio signal follows this pattern:

- From the master the analog signal comes out of one of the selected audio ports (GPIO6 for us)

- The generated signal is a PWM electrical signal with a certain frequency and DutyCycle set by interface

- The cables of the input audio signal to the amplifier will start from the breadboard, with (for each of the two RCA inputs, which has two terminals) the negative terminal placed common to GND with a GPIO port of the master's GND, the positive terminal instead connected to the GPIO6 output (in single-channel configuration)

- From the amplifier you control the amplification (volume) of the output from the knob

- The two amplifier outputs will be connected parallel to the two speakers: positive and negative terminal of the right channel to the right speaker, vice versa for the left one.

All cables used will go or

- inserted into the screw connectors of the adapters,

- knotted (and welded) to the terminals of the speakers,

- inserted into the spring connectors of the amplifier output

- Inserted into the holes of the breadboard. This is a bit more difficult since they are a bit thick hollow